# How to Clean a Commercial Coffee Machine: Deliming the BUNN CWTF-DV-3 Spray Head and Faucet

This SOP explains how to clean a commercial coffee machine by deliming the spray head and water faucet on the BUNN CWTF-DV-3 Coffee Brewer. Following this procedure removes mineral deposits that build up over time, restores proper water flow, and keeps the brewer operating safely and hygienically.

A hand removes the spray head from the underside of the coffee brewer.

# Purpose

The objective of this procedure is to remove mineral deposit buildup from the spray head, spray head fitting, and water faucet assembly of the BUNN CWTF-DV-3 Coffee Brewer. Deliming controls the risk of restricted water flow, faucet leaks, and residue contamination that can occur if mineral scale is left unaddressed.

A hand rotates and moves the deliming spring tool back and forth in the spray head fitting.

# Scope

This SOP applies to the following components of the BUNN CWTF-DV-3 Coffee Brewer:

Brew funnel

Spray head and spray head fitting

Water faucet assembly (handle, bonnet nut, seat cup, safety clip, and aerator)

Exclusion: Any investigation or repair work beyond the faucet assembly (referred to as "beyond FOSA") is outside the scope of this SOP and must be performed by a qualified service technician.

# Supplies

Spray head cleaning tool

Deliming spring tool

New faucet seat cup

Damp cloth

Hand tool (may be needed the first time to loosen the bonnet nut)

Running water source for rinsing the brew funnel and spray head

Measuring cup (used to check faucet flow during the final flush)

The deliming spring tool is inserted into the spray head fitting on the brewer.

Note: The source procedure does not specify chemical delimers, PPE, dilution ratios, or storage requirements; these should be confirmed against the manufacturer's product documentation before performing this task.

# Safety Precautions

[ ] Disconnect power from the brewer before beginning any maintenance.

[ ] Allow the faucet and brewer to cool to the touch; this may take several hours.

[ ] Disconnect the water source before starting the faucet deliming procedure.

[ ] Handle the faucet assembly with caution, as residual water inside may still be hot.

[ ] Do not attempt disassembly or troubleshooting beyond the faucet assembly (FOSA). Doing so risks personal injury and may void the warranty.

[ ] Contact a qualified service technician for any issue that cannot be resolved by cleaning or replacing the faucet assembly.

A person holds the black brew funnel under a running faucet in a white sink, cleaning it thoroughly.

# Cleaning Steps

## Prepare the brewer

Disconnect power and allow to cool

Disconnect the power from the brewer.

Allow the faucet and brewer to cool to the touch, which may take several hours.

A hand operates the hot water faucet lever on the coffee brewer to drain remaining water.

## Clean the spray head assembly

Remove and clean the brew funnel

Remove the entire brew funnel from the brewer.

Thoroughly clean the brew funnel under running water to remove any coffee residue or buildup.

Remove and rinse the spray head

Remove the spray head from the underside of the brewer.

Thoroughly rinse the spray head under running water.

Ensure all holes in the spray head are open and clear of mineral deposits.

A hand holds the spray head and inserts the short end of the cleaning tool into the holes.

Clean the spray head water outlets

Insert the short end of the spray head cleaning tool into each water outlet (hole) of the spray head.

Remove any mineral deposits from each outlet.

A hand reinstalls the spray head onto the fitting under the brewer.

Clean the spray head fitting

Insert the deliming spring tool into the spray head fitting on the brewer.

Rotate the tool several times and pull it back and forth to remove mineral deposits from the fitting.

Wipe the spray head panel

Wipe the spray head panel clean with a damp cloth to remove any remaining residue.

Reinstall the spray head

Reinstall the spray head onto the spray head fitting.

## Delime the water faucet

Disconnect the water source

Disconnect the water source before proceeding with the faucet deliming procedure.

Drain remaining water

Open the faucet and run water until the flow stops completely.

Remove the faucet aerator and safety clip

Remove the faucet aerator and safety clip as the next step in the deliming process.

A close-up of the coffee brewer control panel highlights the requirement to disconnect the water source before this procedure.

Remove the handle and bonnet nut assembly

Remove the handle and bonnet nut assembly from the faucet.

The bonnet nut is typically hand-tight, but a hand tool may be needed the first time.

Exercise caution, as any residual water in the faucet may be hot.

The faucet handle and bonnet nut have been removed from the brewer, exposing the faucet stem.

Exercise caution while accessing the faucet stem

Be careful when removing the assembly, as hot water may still be present in the faucet.

Remove the old faucet seat cup

Remove the old faucet seat cup from the faucet handle stem.

Install the new seat cup

Install a new seat cup onto the faucet handle stem.

Ensure the new seat cup is properly fitted and seated.

Reassemble the faucet components

Replace the faucet components (handle, bonnet nut, etc.) onto the brewer.

Rotate the bonnet nut clockwise and tighten it by hand until secure.

Replace the faucet safety clip and aerator

Reinstall the faucet safety clip and aerator to complete the assembly.

A close-up shows hands installing a new black seat cup onto the faucet handle stem assembly.

A hand reinstalls the faucet handle and bonnet nut onto the brewer, turning the nut clockwise.

A hand holds the faucet safety clip and aerator, preparing to reinstall them onto the faucet.

## Final reset

Wipe down the brewer

Use a damp cloth to wipe down the exterior of the brewer, removing any residue or water spots.

Restore power to the brewer

Reconnect the power supply to the brewer.

Flush the faucet and check for leaks

Run water through the faucet until no signs of dislodged mineral deposits are visible.

Check carefully for leaks around the faucet assembly.

The faucet handle and bonnet nut are removed, exposing the faucet stem. On-screen text: "Exercise caution, as any residual water may be HOT."

A hand wipes the exterior of the BUNN brewer with a white damp cloth.

A hand operates the red hot water faucet lever, running water into a measuring cup to flush out mineral deposits.

# Inspection

[ ] Confirm all holes in the spray head are open and clear of mineral deposits.

[ ] Confirm the spray head is securely reinstalled onto the fitting with no gaps.

[ ] Confirm the faucet assembly shows no leaks after reassembly.

[ ] Confirm water flow through the faucet is steady and free of dislodged mineral debris.

Failed-inspection response: If low water flow persists from the faucet after this maintenance, do not continue troubleshooting. A qualified service technician must remove mineral deposit buildup from the tank outlet fitting and the water path to the faucet assembly. Any further investigation beyond the faucet assembly (FOSA) must be referred to a qualified service technician.

A front view of the BUNN CW Series coffee brewer shows the red faucet handle attached, ready for the handle and bonnet nut assembly to be removed.

A hand continues to operate the faucet lever as water flows into a measuring cup during the flow check.

# Frequency

The source procedure does not specify a fixed daily, weekly, or monthly deliming schedule; the interval for this deliming task should be set according to manufacturer guidance based on local water hardness and brewer usage volume.

BUNN logo with copyright notice: "2016© Bunn-O-Matic Corporation. All Rights Reserved." on a black background.

# What's Next

Contact BUNN customer support or an authorized service provider if you require assistance beyond the faucet assembly, and provide the model number and a description of the issue for faster service.

Refer to the official BUNN documentation or website for additional maintenance information on the CWTF-DV-3 Coffee Brewer.

A front view of the BUNN CWTF-DV-3 coffee brewer with a glass decanter in place shows the reminder that further investigation beyond the faucet requires a qualified service technician.