# How to Create a Control Plan for Electronic Product Assembly

A control plan is the reference document that ties every manufacturing step to its inspection method, sampling rule, and reaction plan. This guide walks through how to create a control plan for an electronic product assembly line, following the manufacturing flow from incoming goods inspection through pre-assembly, final assembly, packing, and outgoing inspection. Use it as a reference when building or reviewing a control plan for your own assembly process.

Control plan table showing document control data: revision level, author, date, and description of change.

# Prerequisites

Before you build the control plan, have the following ready:

Product specifications for each critical component (battery, PCBA, motor, etc.)

A list of equipment and tools used at each process step

Applicable sampling standards (for example, AQL tables)

Inspection record forms: IQC report, first piece inspection record, inline inspection record, PQC inspection record, and OQC inspection record

# Header

The control plan header identifies the plant, customer, program, part number, revision level, plan type, and approval status. The source material for this guide shows only the revision-level fields (revision "A1," authored by AlexChen, dated Dec. 17, 2019, with a description of changes). Plant, customer, program name, part number, plan type, and approval status are not shown and must be filled in with your project's actual details when you create the plan.

# Process flow

Start by reviewing the overall structure of the control plan. Each row corresponds to a specific process step and is organized by process step, characteristic, method, sampling, and control/reaction plan. The columns typically include process number, description, equipment/tool, product, process, special characteristics, production specification/tolerance, evaluation/measurement technique, sampling quantity, frequency, control method, and reaction plan.

The assembly flow covered in this example includes the following operations, in sequence:

| Operation | Process name | Equipment/tool | Linked PFMEA row |
| --- | --- | --- | --- |
| 1 | Incoming goods inspection | Manual visual check, multimeter, comprehensive tester, calipers | Not specified in source |
| 2 | Soldering on PCBA | Soldering iron | Not specified in source |
| 3 | Main board EVA sticking | Tweezers | Not specified in source |
| 4 | Shell/nut mounting | Hand die cutting machine / bottom mold of shell | Not specified in source |
| 5 | Lock PCBA board | Electric screwdriver/fixture | Not specified in source |
| 6 | Pre-final assembly sub-assembly check | Manual comparison against reference samples | Not specified in source |
| 7 | Semi-finished product inspection | Visual and instrument inspection | Not specified in source |
| 8 | Glue application | Glue equipment (air pressure/speed control) | Not specified in source |
| 9 | Battery cover assembly | Electric screwdriver | Not specified in source |
| 10 | Finished product inspection | Visual and functional check, air gun | Not specified in source |
| 11 | Packing | Blister box, color box | Not specified in source |
| 12 | Outgoing inspection | Random sampling per checklist | Not specified in source |

Control plan table listing process numbers, descriptions, equipment/tools, product and process fields, special characteristics, specifications, measurement techniques, sampling, frequency, control method, and reaction plan.

Before starting final assembly, stop the line and check key sub-assemblies. For example, test the push-button for correct hand feel by comparing it against a known good sample, and, where available, reference samples that are too hard or too soft.

During packing, the flow moves from final inspection into packing steps, covering appearance inspection, blister box/battery placement, cleaning, and folding of the color box.

Control plan table showing the transition from inspection to packing, including appearance inspection, blister box/battery placement, cleaning, and color box folding.

# Product characteristics

The following characteristics are checked at incoming inspection and carried through the control plan. Special characteristics are classified using letters:

A – Critical to quality, requiring very specific actions

B – Functional dimension that needs monitoring

C – Attribute requiring monitoring with Statistical Process Control (SPC)

D – Function inspection required

| Characteristic | Specification/tolerance | Classification | Measurement method |
| --- | --- | --- | --- |
| General appearance | No dirt, shrinkage, or glue shortage | Not specified | Manual visual inspection |
| Battery voltage | Higher than 1.61V | Not specified | Multimeter (instrument testing) |
| Battery appearance | Mark intact and undamaged | Not specified | Visual |
| PCBA parts | No missing or losing electronic parts | Not specified | Visual (100% inspection) |
| PCBA QC pass sticker | Passed test | Not specified | Visual (100% inspection) |
| PCBA packaging | Packed in antistatic box | Not specified | Visual (100% inspection) |
| Motor speed | 6500 ± 10% RPM | Not specified | Instrument testing |
| Motor current | 1.5V working voltage, current < 345mA | Not specified | Instrument testing |
| Motor voltage | Starting voltage 0.4V, rated voltage 1.5V | Not specified | Instrument testing |
| Motor service life | At least 500h | Not specified | Instrument testing |
| Motor noise | < 60dB under rated voltage | Not specified | Instrument testing |
| Motor size | 30 × 15.2 × 11.8 mm | Not specified | Calipers |
| Motor appearance | No oxidation/dirt, mark intact | Not specified | Visual |
| Battery cover screw | Tightened, no sliding wire, lock not deviated | A/D (special characteristic) | Visual, instrument (torque) |

Control plan table showing incoming goods inspection, battery testing, PCBA inspection, and motor inspection, with columns for characteristics, methods, sampling, and reaction plans.

Control plan table defining special characteristics A, B, C, D alongside document control data.

# Process characteristics

Process-level parameters are set for each pre-assembly and assembly operation, with defined limits and inspection frequency.

| Process parameter | Target setting | Limit | Monitoring frequency | Responsible role |
| --- | --- | --- | --- | --- |
| Soldering iron temperature | 350°C | ± 30°C | 5 pieces every 2 hours | Line leader or quality inspector |
| Welding spot time | 2–3 seconds | N/A | 5 pieces every 2 hours | Line leader or quality inspector |
| Main board EVA sticking | No skew, no LED occlusion | N/A | 5 pieces every 2 hours | Line leader or quality inspector |
| Shell/nut mounting | Nut pressed to bottom, no shell damage | N/A | 5 pieces every 2 hours | Line leader or quality inspector |
| PCBA screw torque | 2.0–2.5 kgf.cm | No damage to screw or PCBA | 5 pieces every 2 hours | Line leader or quality inspector |
| Glue application (setting 1) | Air pressure 0.15–0.2 MPa | Speed 20–25 | 5 pieces every 2 hours | Line leader or quality inspector |
| Glue application (setting 2) | Air pressure 0.18–0.25 MPa | Speed 26–30 | 5 pieces every 2 hours | Line leader or quality inspector |
| Glue application (setting 3) | Air pressure 0.22–0.35 MPa | Speed 31–35 | 5 pieces every 2 hours | Line leader or quality inspector |
| Battery cover screw torque | 2.5–3.0 kgf.cm | No sliding wire, lock deviation | 5 pieces every 2 hours | Line leader or quality inspector |

Control plan table showing glue application parameters (air pressure, speed), assembly steps such as massage brush installation and battery cover assembly, with special characteristics and torque values.

Operators must wear static rings before soldering and EVA sticking operations. Torque values and known failure modes (missed screws, non-functioning push-button, stuck parts) should be included in operator work instructions so that operators understand the process specifications and inspection criteria they are working to.

# Methods

Inspections use a mix of visual checks, instrument testing, and full or sampled inspection, depending on the process step.

Incoming goods: Sampling GII, AQL 0.6/1.0, each lot; method is manual visual inspection or instrument testing (multimeter, comprehensive tester, calipers); results recorded in the IQC report.

PCBA inspection: 100% full inspection; visual method; results recorded in the IQC report.

Pre-assembly and assembly steps (soldering, EVA sticking, shell/nut mounting, PCBA locking, glue application, battery cover assembly): Sample 5 pieces every 2 hours; visual or instrument method; results recorded in the first piece inspection record and inline inspection record.

Semi-finished product inspection: 100% full inspection covering key test, LED test, working mode test, appearance, and noise test; results recorded in the PQC inspection record.

Finished product inspection: 100% inspection by a dedicated inspector, covering key test, LED brightness, working mode, appearance, noise, waterproofing, and removal of battery/dust; results recorded in the PQC inspection record.

Outgoing inspection: Random sampling, GII, AQL 0.65/1.0, 2 pieces per lot for product checks and 1 box per lot for packaging; results recorded in the OQC inspection record.

Control plan table showing soldering iron use, static ring requirements, temperature/time settings, and inspection methods for pre-assembly operations.

Control plan table showing semi-finished product inspection steps: key test, LED check, working mode test, appearance, and noise test, with 100% inspection and record requirements.

Calibration requirements for gauges and instruments (multimeter, comprehensive tester, calipers) are not detailed in the source material and should be defined separately in your gauge management procedure.

During packing, additional checks are performed: inspect the appearance of the finished product for cleanliness, confirm the blister box and batteries are free from damage and foreign matter, clean the product surface with an air gun, and inspect the color box for damage, dirt, or dust before sealing.

Outgoing inspection goes beyond the 100% in-line checks, applying random sampling for a deeper review of appearance, key function, LED brightness, working mode, noise, waterproofing, drop test, packaging method, identification, and weight.

The outgoing inspection checklist specifies exact criteria for each attribute, such as "no dirt or other appearance defects," "check push-button hand feel," "LED brightness," "no noise or abnormal noise, ≤ 58dB," "waterproof test for 30 minutes," and "drop test from 1m," along with the applicable sampling quantity and frequency.

Control plan table highlighting finished product inspection: key test, LED, working mode, appearance, noise, waterproof test, and air gun cleaning, with 100% inspection and reaction plan for defects.

Control plan table detailing packing steps: appearance inspection, blister box placement, cleaning, folding color box, and sealing, with inspection methods and reaction plans.

Control plan table for outgoing inspection listing appearance, key test, LED, working mode, noise, waterproof test, drop test, packaging, identification, and weight, with random sampling and OQC record.

Control plan table showing the detailed outgoing inspection checklist, including criteria, sampling, and documentation requirements.

# Reaction plan

The reaction plan defines what happens when a check fails at any stage.

Immediate containment: For incoming goods, sort out the non-conforming item and return it to the supplier. For in-process defects (soldering, EVA sticking, shell/nut mounting, PCBA locking, glue application, battery cover assembly), rework or scrap the item.

Escalation: Inspections are performed by the production line leader or by an external inspector from the quality department; either role can raise a non-conformity for action.

Disposition: Place defective finished products in a designated red box (or similar container) so they are not mixed with conforming products, pending rework or scrap decision.

Restart criteria: Perform a first piece inspection immediately after starting assembly. If problems are found, stop and correct them before continuing mass production. Routine in-process inspection of 5 pieces every 2 hours then continues for all critical assembly steps.

Control plan table showing inspection frequency (5 pieces every 2 hours), responsible parties, and documentation requirements for pre-assembly steps.

Special characteristics marked "A" or "D" are treated as critical to product quality — for example, the battery cover screw torque — and any deviation follows the same rework-or-scrap reaction path documented in the first piece inspection record and inline inspection record.

Control plan table showing continued packing steps and reaction plans for non-conforming products, emphasizing designated containers for defective items.

# Revision history

Document control ensures only the latest, approved version of the control plan is used in production and inspection, and it provides traceability for changes.

| Revision | Reason for change | Approver | Effective date |
| --- | --- | --- | --- |
| A1 | Added process for outgoing inspection and equipment parameters for glue equipment, etc. | AlexChen | Dec. 17, 2019 |

Keep documents stored securely and organized to prevent loss or unauthorized changes, and review them regularly as processes or requirements change. Always refer to the current version of the control plan and its associated checklists during operations.

# What's next

Once your control plan is drafted, link each process row to its corresponding PFMEA entry, confirm gauge calibration schedules with your quality team, and train operators and inspectors on the sampling frequency and reaction plan for each step before releasing the plan for production use.

Control plan table showing special characteristics definitions and document control data, summarizing the importance of document control.